# STM32CubeProgrammer CLI — UnixTron Guide

# 1. Why we use it

UnixTron is the orchestrator. It builds firmware; STM32CubeProgrammer performs the actual ST-LINK/SWD programming.

PC → USB → ST-LINK → SWDIO/SWCLK → STM32F401CC → internal Flash.

# 2. Official download

Use STMicroelectronics' official STM32CubeProgrammer page. The current ST page lists the Linux installer and documentation.

Official: https://www.st.com/en/development-tools/stm32cubeprogrammer.html

STM32CubeProgrammer is free but not open source.

# 3. Arch Linux prerequisites

ST documents libusb and udev rules for Linux ST-LINK operation. On Arch Linux install libusb:

sudo pacman -Syu libusb

Do not use Ubuntu's apt command on Arch.

# 4. Linux installation

Download the Linux package from ST and extract it:

unzip <downloaded-package>.zip

cd <extracted-directory>

Find the installer, normally named similar to SetupSTM32CubeProgrammer-2.23.0.linux.

If needed: chmod +x ./SetupSTM32CubeProgrammer-*.linux

Run it: sudo ./SetupSTM32CubeProgrammer-*.linux

Follow the installer and choose the installation directory.

# 5. PATH

The CLI executable is STM32_Programmer_CLI inside the installation's bin directory.

Temporary test: export PATH=<install-directory>/bin:$PATH

Verify: which STM32_Programmer_CLI

Verify version: STM32_Programmer_CLI --version

For persistence, put the export in ~/.bashrc and run source ~/.bashrc.

# 6. Linux USB permissions / udev

ST's Linux manual says the package contains udev rules under Driver/rules for ST-LINK/J-Link/USB DFU use.

Locate them with: find <install-directory> -path '*Driver/rules*' -type f

Install the supplied rules according to the package layout, for example: sudo cp <install-directory>/Driver/rules/* /etc/udev/rules.d/

Then unplug/replug the ST-LINK. If the package has a different path, use the actual Driver/rules location.

# 7. First hardware test

Connect the ST-LINK USB cable.

Run: lsusb

If Linux does not see the ST-LINK, stop here: it is a USB/cable/probe/power problem, not a UnixTron problem.

# 8. Does CubeProgrammer see ST-LINK?

Run: STM32_Programmer_CLI -l

The official CLI reference says -l/--list lists available UART, DFU and ST-LINK interfaces.

# 9. Understand SWD

SWD (Serial Wire Debug) is the two-wire ARM debug/programming interface.

SWDIO = bidirectional debug data.

SWCLK = debug clock.

GND = common electrical reference.

NRST = optional reset connection, useful for hardware-reset/under-reset workflows.

Target power depends on the board/probe. Never assume a 5 V connection is safe for a 3.3 V STM32 target.

# 10. Test the target connection

After ST-LINK is detected, run: STM32_Programmer_CLI -c port=SWD

CubeProgrammer's -c/--connect establishes the target connection. ST documents SWD frequency, reset and connection-mode options.

If normal connection fails, try under-reset when appropriate: STM32_Programmer_CLI -c port=SWD mode=UR

# 11. Manual flash test

Our generated binary is build/firmware.bin.

STM32CubeProgrammer uses -d/-w to download/write a binary and -v to verify it.

Run: STM32_Programmer_CLI -c port=SWD -w build/firmware.bin 0x08000000 -v -rst

Meaning: -c port=SWD connects; -w writes; 0x08000000 is the Flash destination; -v verifies; -rst resets the target.

# 12. Why 0x08000000?

Our Linker.ld places the application in STM32F401CC internal Flash beginning at 0x08000000.

The binary's destination must agree with the linker script and the MCU memory map.

# 13. UnixTron integration

Only after the manual CubeProgrammer command works should UnixTron call it.

Desired flow: ut flash → Flasher → Process::run() → STM32_Programmer_CLI → ST-LINK/SWD → STM32F401CC.

Eventually board, programmer, image path and Flash address should come from unixtron.toml rather than being hard-coded.

# 14. Debug decision tree

CLI not found → installation/PATH.

lsusb does not show ST-LINK → USB/cable/probe/power.

lsusb sees ST-LINK but CLI -l does not → udev/permissions/CubeProgrammer installation.

CLI sees ST-LINK but SWD connection fails → SWD wiring, target power, reset or target state.

Connection succeeds but write fails → protection, target state, address or image problem.

Write succeeds but verify fails → image/address/memory problem.

Programming succeeds but application does not run → firmware/linker/startup/clock/GPIO problem.

# 15. UnixTron status

CLI ✅  Project generation ✅  Config ✅  ARM GCC ✅  main.c ✅  startup.c ✅  linker ✅  ELF/BIN ✅

Current blocker: install STM32CubeProgrammer and establish ST-LINK/SWD connection.

# 16. Exact next checklist

1. sudo pacman -Syu libusb

2. Download the Linux STM32CubeProgrammer package from ST.

3. Run the Linux installer.

4. Add its bin directory to PATH.

5. STM32_Programmer_CLI --version

6. Connect ST-LINK and run lsusb.

7. STM32_Programmer_CLI -l

8. Test STM32_Programmer_CLI -c port=SWD

9. Manually flash build/firmware.bin.

10. Only then finish UnixTron's Flasher command.

# 17. Official references

https://www.st.com/en/development-tools/stm32cubeprogrammer.html

https://dev.st.com/stm32cube-docs/prog/latest/en/index.html

https://dev.st.com/stm32cube-docs/prog/2.23.0/en/docs/markup/CubeProg_Command_Lines.html

https://www.st.com/resource/en/user_manual/dm00403500-stm32cubeprogrammer-software-description-stmicroelectronics.pdf