# UnixTron — Development Path & Current State

Project goal: Build a CLI-first embedded development platform that standardizes the workflow from project creation → build → firmware generation → flashing for an STM32F401CC, with multi-board support considered only after v1.

# 1. Current v1 Target

The v1 definition of done is a CLI workflow that can create a project, compile it, generate a firmware image, flash it to the STM32F401CC through ST-Link, and combine build + flash into a single run command.

ut init <project>

ut build

ut flash

ut run

# 2. Current Repository Structure

unixtron/
├── CMakeLists.txt
├── README.md
├── build/                         # UnixTron executable/build artifacts
│   └── unixtron
│
├── core/
│   ├── build/
│   │   ├── Builder.cpp
│   │   └── Builder.hpp
│   ├── config/
│   │   ├── config.cpp
│   │   └── config.hpp
│   ├── flash/
│   │   ├── flash.cpp
│   │   └── flash.hpp
│   ├── process/
│   │   ├── Process.cpp
│   │   └── Process.hpp
│   └── project/
│       ├── ProjectGenerator.cpp
│       └── ProjectGenerator.hpp
│
├── src/
│   ├── main.cpp
│   ├── cli/
│   └── commands/
│       ├── build_command.cpp
│       ├── build_command.hpp
│       ├── flasher_command.cpp
│       ├── flasher_command.hpp
│       ├── help_command.cpp
│       ├── help_command.hpp
│       ├── init_command.cpp
│       ├── init_command.hpp
│       ├── run_command.cpp
│       ├── run_command.hpp
│       ├── version_command.cpp
│       └── version_command.hpp
│
└── blink/                         # Generated embedded project
    ├── CMakeLists.txt
    ├── README.md
    ├── unixtron.toml
    ├── STM32F401CC_FLASH.ld
    ├── include/
    ├── build/                     # Firmware build artifacts
    └── src/
        ├── main.c
        └── startup.c

# 3. Architecture

UnixTron is an orchestrator, not a replacement compiler or programmer.

CLI → Command Registry → Command → Core service → Process → External tool → MCU

CLI: receives commands such as init/build/flash/run.

Command layer: converts CLI commands into actions.

ProjectGenerator: creates the embedded project structure and initial files.

Config: reads unixtron.toml and exposes board/toolchain/flasher settings.

Builder: coordinates compilation, linking and binary generation.

Flasher: coordinates the STM32 programming tool.

Process: executes external programs such as arm-none-eabi-gcc, objcopy and STM32_Programmer_CLI.

# 4. Build Pipeline

Current intended pipeline:

main.c + startup.c
        ↓
arm-none-eabi-gcc
        ↓
main.o + startup.o
        ↓
GCC linker + STM32F401CC linker script
        ↓
firmware.elf
        ↓
arm-none-eabi-objcopy
        ↓
firmware.bin

# 5. STM32F401CC Project Files

src/main.c — application code; current test toggles GPIOA/PA5.

src/startup.c — reset handler, vector table and basic .data/.bss initialization.

STM32F401CC_FLASH.ld — linker memory layout and sections.

unixtron.toml — project configuration.

# 6. Configuration

Generated unixtron.toml:

name = "blink"
board = "stm32f401cc"
toolchain = "arm-none-eabi-gcc"
flasher = "stlink"
optimization = "Debug"

# 7. Process Layer

Current ProcessResult API:

struct ProcessResult {
    int exitCode;
    std::string standardOutput;
    std::string standardError;
};

Current Linux implementation uses popen()/pclose() and temporarily merges stderr into stdout with 2>&1. This is sufficient for v1 debugging but should eventually be replaced with proper separate stdout/stderr capture.

# 8. What Has Been Successfully Tested

UnixTron itself builds successfully with CMake.

version command works.

help command works.

build command is registered.

arm-none-eabi-gcc is installed at /usr/bin/arm-none-eabi-gcc.

Config successfully reads the generated unixtron.toml.

Builder successfully starts.

main.c compilation succeeds.

startup.c is now generated correctly.

Current blocker: linker stage fails.

# 9. Current Blocker — Linker

The project has successfully reached the linker. Do not change architecture now. The next debugging step is to capture the complete linker error from:

../build/unixtron build

Expected progress before the failure:

[1/3] Compiling main.c... OK
[2/3] Compiling startup.c... OK
[3/3] Linking firmware...

The exact linker error should determine the next fix. Likely areas to inspect are the linker script, startup symbols, section placement, or architecture/linker flags.

# 10. Development Rules Going Forward

Keep the current repository structure; do not rename/move folders unless necessary.

Debug one layer at a time.

Do not add Arduino/ESP32/multi-board support until STM32F401CC v1 works.

Do not add GUI, plugins or serial monitor before v1 works.

Test every layer before moving upward.

# 11. Immediate Next Steps

1. Fix the STM32F401CC linker error.

2. Confirm build/firmware.elf is generated.

3. Generate and confirm build/firmware.bin.

4. Implement/test Flasher with ST-Link + STM32_Programmer_CLI.

5. Implement/test ut flash.

6. Implement/test ut run = build + flash.

7. Declare UnixTron v1 complete after real STM32 execution is verified.

# 12. Useful Commands

cmake --build build

./build/unixtron version

./build/unixtron help

./build/unixtron init blink

cd blink

../build/unixtron build

which arm-none-eabi-gcc

arm-none-eabi-gcc --version

arm-none-eabi-objcopy --version

Status: UnixTron v1 is in the final build/link/flash phase.